MILITARY FIRMWARE FORKS

Combat-Adapted Open-Source Drone Firmware

Handbook Chapter + Forge Database Reference

Generated: 2026-04-04

Drone Integration Handbook • forgeprole.netlify.app

CONFIDENTIAL — INTERNAL USE

// THE LANDSCAPE

The Russo-Ukrainian War has produced the fastest firmware innovation cycle in aviation history. Open-source drone firmware—originally built for hobbyist racing quads—has been forked, hardened, and deployed at industrial scale across the battlefield. Both sides now operate custom firmware ecosystems that transform $300 commercial hardware into EW-resistant strike platforms.

This chapter maps every known military firmware fork, its lineage, capabilities, and relevance to the Forge/Wingman ecosystem. Cross-references: Ch.2 (RF Fundamentals), Ch.5–7 (Firmware), Ch.8 (UART/Serial).

## Master Comparison

| Firmware | Base | Origin | Layer | Status | Key Capability |
| --- | --- | --- | --- | --- | --- |
| MILELRS | ELRS 3.3.2 | Ukraine | Control Link | ACTIVE | Multiband, encrypted, EW scanning, swarm ID |
| MILBETA | BF 4.3/4.5 | Ukraine | FC Firmware | ACTIVE | EW OSD overlay, VTX 3–7 GHz, extended failsafe |
| MafiaLRS | ELRS | Ukraine | Control Link | ACTIVE | 433–735 MHz non-standard bands, freq hopping |
| BarvinokLRS | Custom | Ukraine | Control Link | ACTIVE | Free distro to frontline, EW-resistant |
| Barvinok-5 | Custom | Ukraine | Video Link | PROTOTYPE | Encrypted digital video, FHSS, replaces analog |
| "1001" | DJI OEM | Russia | Platform FW | DISRUPTED | DJI unlock: flight limits, GPS spoof resist, batteries |
| CIAJeepDoors | DJI OEM | Ukraine | OPSEC Patch | ACTIVE | Disables DJI Remote ID / AeroScope tracking |
| mLRS | Custom LoRa | Open Source | Control+Telem | ACTIVE | MAVLink RC+telemetry, 7–87 km, diversity |
| DroneBridge | ESP-IDF | Open Source | Telemetry | ACTIVE | ESP32 serial→WiFi/ESP-NOW, AES-256, swarm |

// FIRMWARE PROFILES

## MILELRS

Base: ExpressLRS 3.3.2

Origin: Ukrainian engineers, first half 2024

Current Version: v2.30

Distribution: Access-controlled via Telegram (@dankzb.03), unique TX binding key required

MILELRS is the most widely deployed military control link firmware in the Russo-Ukrainian War. Designed as a drop-in replacement for stock ExpressLRS, it runs on the same COTS hardware (HappyModel, BETAFPV, RadioMaster, etc.) but adds a suite of EW-hardening features that transform hobby receivers into contested-environment comms systems.

### Capabilities

Multiband Redundancy: Simultaneous operation across 900 MHz and 2.4 GHz using diversity receivers. If one band is jammed, the other maintains link.

Encrypted Binding: Unique per-transmitter key required for binding. Prevents unauthorized connection to friendly drones.

EW Scanning: Real-time spectrum scanning that detects jamming sources and reports direction/strength to the FC via telemetry.

Auto Channel Switching: Automatic frequency hopping on interference detection. Operator can also manually configure frequency ranges.

Multi-RX Support: Multiple receiver modules can connect to a single FC for redundant control paths.

Swarm ID: Unique drone identification within coordinated multi-unit operations.

Failsafe Extensions: Extended failsafe timers and configurable behaviors for signal loss in combat conditions.

### Supported Hardware

TX: HappyModel ES900/ES24, BETAFPV 900/2.4 GHz, RadioMaster Bandit/Ranger, EMAX OLED, CYCLONE

RX: HappyModel ES900/Dual, BETAFPV SuperD, RadioMaster BR1/BR3, GEPRC, Foxeer, NeutronRC, iFlight, BAYCKRC, HGLRC Hermes, EMAX, AION Mini

### MILBETA Synergy

MILELRS is designed to pair with MILBETA. The RX module sends detailed EW telemetry (packet loss by frequency band, jamming source direction, LQ statistics) to the FC, which MILBETA renders on the pilot’s OSD. Functions 3 and 6 in MILBETA (EW direction-finding and LQ statistics display) require MILELRS firmware on the receiver.

### Operational Deployment

By summer 2024, MILELRS was already among the most common militarized control systems in Ukraine. Russia gained access to an earlier version by 2025 and adopted it. TAF Industries ships the Kolibri 10 strike drone with MILELRS pre-installed. Production-scale deployment confirms it has moved beyond experimental use into standard military issue.

### Cross-References

Handbook Ch.2 (RF Fundamentals) — frequency bands, hopping strategies

Handbook Ch.5 (ELRS) — base protocol architecture

Handbook Ch.8 (UART/Serial) — FC-to-RX telemetry wiring

Forge: MafiaLRS firmware generator — related but distinct fork

## MILBETA

Base: Betaflight 4.3.3 / 4.5.1

Origin: Ukrainian engineers, paired with MILELRS

Current Version: v1.23

Distribution: Via Telegram, no license required

MILBETA is a Betaflight fork purpose-built to process and display tactical data from MILELRS receivers. While stock Betaflight is designed for flight performance tuning, MILBETA adds a combat information layer that turns the pilot’s OSD into a tactical display.

### Capabilities

1. OSD Camouflage: Hides launch and landing site coordinates from the OSD to prevent intelligence gathering if video feed is intercepted.

2. Switch Position Display: Shows current toggle switch positions on OSD for rapid mode awareness.

3. EW Direction Finding: Displays jamming source direction and signal strength on OSD. Requires MILELRS synergy.

4. Extended Failsafe: All failsafe timers multiplied by 10x. Writing 1 sec in Betaflight Configurator = 10 sec actual. Allows drone to continue mission through temporary signal loss.

5. VTX Frequency Unlock: Removes stock frequency restrictions. Stock BF: 4900–5999 MHz. MILBETA: 3000–6999 MHz. Enables operation on non-standard video bands that enemy EW may not cover.

6. EW + LQ Statistics: Real-time display of electronic warfare signals and link quality across 12 frequency bands. Requires MILELRS.

7. Armed VTX Switching: Allows VTX channel changes while motors are armed and in flight.

### FC Compatibility

BF 4.3.3 recommended for older FCs (F722 and similar). BF 4.5.1 for newer FCs (F435). All standard Betaflight targets are supported—MILBETA is a firmware-level modification, not a hardware change.

### Cross-References

Handbook Ch.6 (Betaflight) — base firmware architecture, CLI, OSD

Handbook Ch.3 (Antennas/Video) — VTX frequency planning

Forge: Troubleshoot — failsafe diagnostics

## FPV_VYZOV Ecosystem

Affiliation: Russian non-state developer network

Components: MILELRS + MILBETA + custom hardware designs

Distribution: Telegram channels with weekly “Friday Updates”

FPV_VYZOV (“Challenge”) is a Russian-affiliated developer collective that has built a complete tactical FPV ecosystem around MILELRS and MILBETA. They adopted an earlier Ukrainian version of MILELRS and extended it with their own modifications.

### Key Innovation: EW Hunter-Killer Drones

The ecosystem’s signature capability is specialized drones that detect, track, and neutralize enemy EW assets. Using MILELRS’s spectrum scanning to locate jamming sources, the hunter-killer drone flies toward the EW emitter while MILBETA displays direction-finding data on the pilot’s OSD. This creates a direct counter to conventional drone defenses.

### Supply Chain Model

Built entirely on COTS hardware from Chinese manufacturers (HappyModel, BETAFPV) procured through Russian domestic online stores (Ozon, Wildberries). Combined with open-source firmware and 3D-printed hardware designs, this model allows faster iteration than traditional military acquisition.

## MafiaLRS

Base: ExpressLRS

Origin: Ukrainian FPV community

Status: Active — 254 RX / 122 TX targets in Forge DB

Source: BUSHA/targets@mafia-targets (GitHub)

MafiaLRS is a separate Ukrainian ELRS fork (distinct from MILELRS) that operates on non-standard frequency bands: 433 MHz, 490–560 MHz, and modified 868/915 MHz. The key difference from stock ELRS is operation outside standard EW jamming coverage—most Russian jammers target the well-known 868/915 MHz and 2.4 GHz ELRS bands. MafiaLRS sidesteps this by operating in bands that aren’t covered.

Forge has a self-hosted firmware generator on the Tools page with full target browser. The external dependency on vtx.in.ua has been eliminated.

### Cross-References

Forge: Tools page — MafiaLRS firmware generator

Forge: Vault — grey-area combat components

Handbook Ch.2 — frequency band planning

## Barvinok-5 / BarvinokLRS

Origin: Ukrainian startup (6-person team, mostly part-time)

Products: BarvinokLRS (control link, free), Barvinok-5 (digital video, prototype)

Funding: Defense Builder accelerator grant

### BarvinokLRS (Control Link)

A custom control firmware distributed free to Ukrainian frontline units. Not a fork of ELRS—it’s independently developed. Operates on multiple frequencies with different RF parameters. Field reports confirm it maintains link where other systems fail under EW pressure.

### Barvinok-5 (Video Module)

A digital video system designed to replace analog FPV video while keeping its advantages (one-directional transmission, low latency). Adds encryption and wide-spectrum pseudo-random frequency-hopping modulation (FHSS) with very small packets, making the signal extremely difficult to jam or intercept. Approaching end of prototype phase, beginning integration with manufacturers and military units.

### Why This Matters

Analog video is the weakest link in FPV drone operations. It’s unencrypted (interceptable by any receiver) and easy to jam. Video jamming became the most effective Russian counter-drone tactic starting summer 2024. Barvinok-5 is one of several Ukrainian attempts to solve this—others include open-source approaches like HDZero, Walksnail, and frequency-band diversification (operators now use 1.2 GHz, 5.8 GHz, and even 7.2 GHz video).

## DJI Military Modifications

### "1001" Firmware (Russian)

Platform: DJI Mavic series and similar consumer drones

Distribution: Closed network of “terminals” (pre-configured laptops) at drone service centers

Scale: ~200,000 drones flashed as of March 2025

Status: Distribution infrastructure compromised by cyberattack (July 2025)

Removes manufacturer-imposed flight limits, improves GPS spoofing resistance, and enables high-capacity battery support. The firmware itself was reportedly not compromised in the attack, but the terminal-based update system was disabled, preventing new drones from being flashed.

### CIAJeepDoors (Ukrainian)

Platform: DJI drones with Remote ID

Delivery: USB device called “Olga”

Activates a command in DJI firmware that disables Remote ID broadcasting. Remote ID allows Russian AeroScope terminals to identify both the drone’s position and the operator’s position. With Remote ID active, Russian artillery can target drone operators. CIAJeepDoors closes this vulnerability. DJI has since released firmware on newer models that prevents Remote ID from being disabled.

## Open-Source Building Blocks

### mLRS

Base: Custom LoRa firmware (SX126x / SX128x / STM32WLE5)

Bands: 433 MHz, 868/915 MHz, 2.4 GHz

Range: 7–87 km depending on mode and power

Protocol: Integrated RC + full MAVLink telemetry

mLRS is not a military fork, but its architecture—combined RC and MAVLink telemetry over LoRa with full diversity support—makes it directly relevant to BVLOS operations and contested environments. MatekSys manufactures purpose-built mLRS hardware (mR900-30, mR24-30). Also runs on repurposed ELRS and FrSky R9 hardware. Supported natively by ArduPilot.

### DroneBridge ESP32

Base: ESP-IDF (pure C, not Arduino)

Protocols: MAVLink, MSP, LTM, transparent serial

Encryption: AES-256-GCM in all modes including ESP-NOW broadcasts

Range: ~1 km (ESP-NOW LR with external antenna)

Ultra-low-cost ($3–5 per board) serial-to-WiFi/ESP-NOW telemetry bridge. Supports swarm networking via ESP-NOW with encrypted broadcasts. Useful as a cheap telemetry fallback or local mesh relay. Special Skybrush-optimized version exists for drone shows. Not a combat system, but a building block for low-cost telemetry architectures.

// BROADER CONTEXT

## The EW Arms Race

The firmware fork ecosystem exists because of the escalating EW arms race in Ukraine. The cycle runs: drones use standard frequencies → enemy deploys jammers on those frequencies → firmware forks move to non-standard frequencies → enemy expands jamming coverage → firmware adds frequency hopping and encryption → enemy develops wideband jammers → firmware adds multiband redundancy and auto-switching → video jamming emerges as the new weak point → encrypted digital video systems develop.

By summer 2025, Ukraine deployed video on the 7.2 GHz band—causing significant disruption because Russian jammers weren’t covering it. Both sides now compile and share statistics on enemy video frequencies detected by ELINT forces, feeding a centralized competition of constant tactical innovation.

## Production Scale

Ukraine’s defense industry can produce 8+ million FPV drones per year across 160+ manufacturers. FPV drones now account for 60% of Russian combat losses. TAF Industries alone produces 80,000 drones/month. Russia’s fiber-optic FPV production reached 50,000+/month by September 2025. This is the context in which firmware forks operate—they’re not experimental projects, they’re production firmware for industrial-scale drone warfare.

## Fiber-Optic: The EW Endgame

Fiber-optic FPV drones are immune to all RF-based EW because the control signal travels through a physical cable, not over the air. Both sides are now mass-producing fiber-optic FPVs. Russia has domestic fiber cable production at the Saransk plant and Chinese spool cooperation. Ukraine codified and scaled fiber-optic variants in 2025. The Kolibri 13 O (fiber-optic variant) is one example. Fiber-optic drones make all RF-layer firmware hardening irrelevant—but they have range and weight limitations, so RF-based systems remain necessary for many mission profiles.

// FORGE DATABASE ENTRIES

The following entries should be added to forge_database.json under a new “military_firmware” category, or distributed across existing categories (receivers, control_link_tx, firmware) with a “military_fork” tag.

| Name | PID | Category | Origin | Notes |
| --- | --- | --- | --- | --- |
| MILELRS | MFW-0001 | military_firmware | Ukraine | ELRS 3.3.2 fork, multiband, encrypted, EW scanning |
| MILBETA | MFW-0002 | military_firmware | Ukraine | BF fork, EW OSD, VTX unlock 3–7 GHz, failsafe 10x |
| BarvinokLRS | MFW-0003 | military_firmware | Ukraine | Independent control link, free distro to military |
| Barvinok-5 Video | MFW-0004 | military_firmware | Ukraine | Encrypted FHSS digital video, analog replacement |
| "1001" DJI Mod | MFW-0005 | military_firmware | Russia | DJI unlock, GPS spoof resist, 200K+ drones |
| CIAJeepDoors | MFW-0006 | military_firmware | Ukraine | DJI Remote ID disable, delivered via Olga USB |
| mLRS | MFW-0007 | military_firmware | Open Source | LoRa RC+MAVLink, 7–87 km, MatekSys HW |
| DroneBridge ESP32 | MFW-0008 | military_firmware | Open Source | ESP32 telem bridge, AES-256, swarm ESP-NOW |

// HANDBOOK INTEGRATION PLAN

This content maps to the Drone Integration Handbook as follows:

### New Chapter: Military Firmware Forks

File: components/military-firmware.md

Location: After existing firmware chapters (Ch.5–7), before field ops (Ch.10+)

Content: This document’s Landscape + Profiles sections, adapted to markdown

### Cross-Reference Updates

Ch.2 (RF Fundamentals): Add section on non-standard frequency operation and EW evasion bands (433 MHz, 490–560 MHz, 7.2 GHz video)

Ch.5 (ELRS): Add note about military forks (MILELRS, MafiaLRS) and their divergence from stock ELRS

Ch.6 (Betaflight): Add note about MILBETA and combat-specific OSD modifications

Ch.3 (Antennas/Video): Add section on video jamming countermeasures and frequency diversification

### Forge Integration

Browse page: New “military_firmware” category with 8 entries

Tools page: Existing MafiaLRS generator already covers one fork. Consider adding mLRS target browser.

Wingman RAG: This chapter will be indexed as RAG context. MILELRS/MILBETA queries will return relevant entries.

Troubleshoot: Add entries for common MILELRS binding issues, MILBETA OSD configuration, and MafiaLRS frequency selection.

// SOURCES

Armada International, “The Curran Papers No. 5: Jamming UAV Video Signals” (Feb 2026)

Cyber Shafarat / Treadstone 71, “Russian FPV Drone Tactical Ecosystem – MILELRS – MILBETA – FPV_VYZOV” (Oct 2025)

Cyber Shafarat, “MILELRS v2.30 firmware introduces substantial upgrades” (Apr 2025)

Cyber Shafarat, “MILBETA v1.23 firmware flashing” (Nov 2024)

IEEE Spectrum, “Ukraine Is the First Hackers’ War” (Mar 2025)

IEEE Spectrum, “Ukraine Weapons Tech Thrives Amid Conflict” (Nov 2025)

Ukraine’s Arms Monitor, “TAF Industries: 150,000 FPVs per Month” (Oct 2025)

The Record / Recorded Future, “Cyberattack deals blow to Russian firmware” (Jul 2025)

dev.ua, “Barvinok-5 video module developer interview” (Oct 2025)

C4ISRNet, “How Ukraine learned to cloak its drones” (Oct 2022)

NSDC Ukraine, “Results of Ukraine’s Defense Industry in 2025: FPV Drones” (Jan 2026)

ArduPilot Docs, mLRS integration guide

GitHub: olliw42/mLRS, DroneBridge/ESP32, BUSHA/targets@mafia-targets